DESCRIPTION

Under this item, the contractor shall saw cut existing concrete pavement, asphalt surface course, or asphalt concrete overlay on concrete pavement at the locations indicated on the plans or as ordered by the engineer.

MATERIALS

None specified.

CONSTRUCTION DETAILS

Existing pavement and overlay shall be saw cut perpendicular to the roadway surface along neat lines, and to the depth indicated on the plans and typical sections.  A power saw approved by the engineer shall be used for cutting asphalt surface course and asphalt overlay.  A power saw shall be used for cutting concrete pavement.  After the existing concrete pavement asphalt surface course or overlay has been saw cut through, the contractor may use pry bars, pneumatic tools or other methods approved by the engineer, to pry loose the existing pavement from that pavement which is to remain.  A pavement breaker, under the supervision of the engineer, may be used to break up the pavement to be removed after the pavement has been completely saw cut through and completely free from the pavement to remain.

Care shall be taken not to disturb or damage saw cuts of existing pavement and curbs to remain.

Any existing pavements and curbs not indicated to be removed that are damaged by the contractor’s operations, shall be repaired by the contractor to the satisfaction of the engineer at no additional cost to the county.

No payment will be made for saw cutting which the contractor may choose to do for his/her own convenience.

METHOD OF MEASUREMENT

The quantity to be paid for under this item will be the number of linear feet of saw cutting done in accordance with the plans, typical sections and the directions of the engineer.

Separate payment for sawcutting shall not be made when excavating in an area to be reconstructed, except where noted in the plans.  However, payment for sawcutting will be made when required for excavating in existing pavement areas that are to remain or as ordered by the engineer.

No sawcutting will be measured for payment under this item which the contractor may choose to do for his own convenience.

BASIS OF PAYMENT

The unit price bid per linear foot of saw cutting shall include the cost of all labor, materials, and equipment necessary to complete the work.  Payment for removal and disposal of cut pavement shall be paid for under the appropriate excavation item.

Payment will be made under:

| Item No. | Item | Pay Unit |
| --- | --- | --- |
| C502.5014 | Sawcutting Asphalt Pavement, Concrete Pavement and Asphalt Overlay On Concrete Pavement | LF |