INTEROPERABLILITY CHANNELS

UTAC

| Description | NPSTC ID | Mobile TX (MHz) | Mobile RX (MHz) |
| --- | --- | --- | --- |
| Calling | UCALL40 | 458.2125 | 453.2125 |
| Calling | UCALL40D | 453.2125 | 453.2125 |
| Tactical | UTAC41 | 458.4625 | 453.4625 |
| Tactical | UTAC41D | 453.4625 | 453.4625 |
| Tactical | UTAC42 | 458.7125 | 453.7125 |
| Tactical | UTAC42D | 453.7125 | 453.7125 |
| Tactical | UTAC43 | 458.8625 | 453.8625 |
| Tactical | UTAC43D | 453.8625 | 453.8625 |

VTAC

| Description | NPSTC ID | Channel (MHz) | CTCSS Tone +/- |
| --- | --- | --- | --- |
| Calling | VCALL10 | 155.7525 base/mobile | CSQ/156.7 (5A) |
| Tactical | VTAC11 | 151.1375 base/mobile | CSQ/156.7 (5A) |
| Tactical | VTAC12 | 154.4525 base/mobile | CSQ/156.7 (5A) |
| Tactical | VTAC13 | 158.7375 base/mobile | CSQ/156.7 (5A) |
| Tactical | VTAC14 | 159.4725 base/mobile | CSQ/156.7 (5A) |

VTAC11 & VTAC12 may not be used in PR/VI.  +/- Default operation should be carrier squelch receive, CTCSS transmit.  If the user can enable/disable without reprogramming the radio, the indicated CTCSS tone should also be programmed for receive, and the user instructed how and when to enable/disable.

ITAC

| Description | NPSTC ID | Mobile TX (MHz) | Mobile RX (MHz) |
| --- | --- | --- | --- |
| Calling | 8CALL90 | 821.0125 (806.0125) | 866.0125 (851.0125) |
| Calling - Direct | 8CALL90D | 866.0125 (851.0125) | 866.0125 (851.0125) |
| Tactical | 8TAC91 | 821.5125 (806.5125) | 866.5125 (851.5125) |
| Tactical - Direct | 8TAC91D | 866.5125 (851.5125) | 866.5125 (851.5125) |
| Tactical | 8TAC92 | 822.0125 (807.0125) | 867.0125 (852.0125) |
| Tactical - Direct | 8TAC92D | 867.0125 (852.0125) | 867.0125 (852.0125) |
| Tactical | 8TAC93 | 822.5125 (807.5125) | 867.5125 (852.5125) |
| Tactical - Direct | 8TAC93D | 867.5125 (852.5125) | 867.5125 (852.5125) |
| Tactical | 8TAC94 | 823.0125 (808.0125) | 868.0125 (853.0125) |
| Tactical - Direct | 8TAC94D | 868.0125 (853.0125) | 868.0125 (853.0125) |

The frequency in parenthesis, which is 15 MHz lower, will be the frequency used after rebanding.